ELECTRONIC CASH REGISTERS

CHD 3320/5620

DRIVER DESCRIPTION

Revision history:

| Date |  | Description of modifications | Author |
| --- | --- | --- | --- |
| 11.01.2008 |  | Initial version | NN |
| 21.01.2008 |  | Minor modifications | NN |
| 11.07.2008 |  | Table numbers and formats added. Report numbers added. | VlaR, Alex |
| 22.09.2008 |  | Report formats added. | NN |
| 25.09.2008 |  | Daily financial report format changed (moved RA /PO from tender sub-report to separate sub-reports) | AJ NN |
| 14.10.2008 |  | Driver return code section updated. | NN |
| 03.11.2008 |  | Department type field added | VlaR |
| 15.11.2008 |  | RA / PO operation name sections added | NN |
| 27.11.2008 |  | Tender section updated | NN |
| 16.12.2008 |  | Fiscal printer mode commands added | NN |
| 18.12.2008 |  | Minor documentation restructure | NN |
| 28.01.2009 |  | Note about CHDVFP at the beginning of fiscal printer section added | VlaR |
| 30.01.2009 |  | Fiscal printer protocol: tender amount, PLU price and department name parameters made optional | NN |
| 06.02.2009 |  | INI-file parameter default values added | NN |
| 30.04.2009 |  | Report clear command related note added | NN |
| 01.06.2009 |  | Missing sections / fields added Transaction start / suspend fiscal commands added | NN VlaR |
| 30.06.2009 |  | Transaction resume fiscal command updated | NN |
| 16.07.2009 |  | Modifier fiscal command updated | NN |
| 04.08.2009 |  | Linked PLU section added | NN |
| 05.08.2009 |  | Daily financial Z1 report clear related note added | VlaR |
| 11.08.2009 |  | Option flag section updated | NN |
| 17.08.2009 |  | Hourly report interval description added | NN |
| 21.08.2009 |  | Printing option section added | NN |
| 15.09.2009 |  | Clerk report section added | NN |
| 16.09.2009 |  | Keyboard layout commands added | NN |
| 23.09.2009 |  | Modifier fiscal command updated Customer report section added | NN NN |
| 15.10.2009 |  | Low level fiscal printer commands removed Transaction log command description added | NN |
| 13.11.2009 |  | New modifier type added – total modifier | NN |
| 01.12.2009 |  | Delete command description added Transaction log format bug fix | NN |
| 07.12.2009 |  | Transaction log sale/return line format enhanced | NN |
| 11.12.2009 |  | Country independent (ISO 8601:2004) date/time format used | NN |
| 18.03.2010 |  | Transaction log sale/return line format enhanced | NN |
| 15.04.2010 |  | Service reminder command data example updated | NN |
| 05.05.2010 |  | COM port, Ethernet, on-line mode and macro key sections added ECR error codes updated | NN |
| 18.05.2010 |  | Moved from option flags to system settings ReceiptPrintEnabled JournalPrintEnabled flags, instead of those 2 option flags introduced ReceiptPrintInRegModeOn JournalPrintInXmodeOn flags | AJ |
| 26.05.2010 |  | Added HALO/LALO field for tender record | AJ |
| 27.05.2010 | Clerk profile field added – “training mode on” |  | AJ |
| 02.06.2010 | Printing option field added – “negative sales total on report” |  | Alex |
| 11.06.2010 | Report arrangement section added Keyboard layout key descriptions added |  | AJ |
| 11.06.2010 | 2 new in-store bar code types added |  | AJ |
| 14.06.2010 | System setting commands added |  | NN |
| 22.06.2010 | Missing option flags added |  | NN |
| 08.07.2010 | Item print options: new flag added |  | NN |
| 15.07.2010 | FTP settings section added |  | NN |
| 30.07.2010 | Financial/clerk report counter descriptions updated |  | NN |
| 09.08.2010 | Item print option added – “print graphical logo after paper cut” Option flag added – “recalculate taxes on daily financial Z1 report” |  | NN Alex |
| 16.11.2010 | New INI-file parameters added |  | NN |
| 25.11.2010 | Added compact tax print flag to option flags |  | AJ |
| 25.07.2011 | Automatic promotion section added Currency section updated |  | NN |
| 01.08.2011 | Added Total barcode section |  | AJ |
| 04.08.2011 | Transaction log section updated |  | NN |
| 25.12.2011 | Refactoring |  | Alex |
| 09.02.2012 | New fields added |  | NN |
| 14.02.2012 | New INI-file parameters added |  | NN |
| 26.03.2012 | Added LogFilePath property description |  | MA |
| 15.10.2012 | Added option flags missing description |  | MA |
| 17.10.2012 | Added printing option flags missing description |  | MA |
| 29.10.2012 | Changed no sale counter to misc. counters and added 2 counters |  | MA |
| 07.11.2012 | New Cashback flag in Tender |  | Alex |
| 17.01.2013 | Added key names |  | AJ |
| 18.01.2013 | Updated tax info |  | AJ |

TABLE OF CONTENTS

1.	Connecting ECR to PC	6

1.1.	Serial cable connection	6

1.2.	Ethernet connection	6

2.	Data preset commands	8

2.1.	Departments	8

2.2.	PLUs	9

2.3.	Linked PLUs	10

2.4.	Tenders	10

2.5.	Discounts / markups	11

2.6.	Automatic promotions	12

2.7.	ECR number	12

2.8.	Option flags	13

2.9.	Date	14

2.10.	Time	14

2.11.	Currencies	15

2.12.	Printing fonts and heights	15

2.13.	Printing options	16

2.14.	COM port settings	17

2.15.	PLU presets	18

2.16.	Clerks	19

2.17.	Clerk profiles	19

2.18.	Graphical logo	20

2.19.	Taxes	20

2.20.	Text header	21

2.21.	Text footer	21

2.22.	Customers	21

2.23.	PLU barcode	22

2.24.	Total barcode	23

2.25.	In store barcodes	23

2.26.	Report arrangement	24

2.27.	Card ranges	25

2.28.	Paid out operation names	25

2.29.	Receive on account operation names	26

2.30.	Macro key	26

2.31.	FTP settings	26

2.32.	Ethernet settings	27

2.33.	On-line protocol settings	28

2.34.	System settings	28

2.35.	Keyboard layout	29

2.36.	Service reminder	31

3.	Report commands	33

3.1.	Daily financial report Z1	33

3.2.	Daily financial report Z2	37

3.3.	Department report Z1	38

3.4.	Department report Z2	38

3.5.	PLU report Z1	38

3.6.	PLU report Z2	39

3.7.	Clerk report Z1	39

3.8.	Clerk report Z2	40

3.9.	Hourly report	40

3.10.	In drawer report	40

3.11.	Customer report Z1	41

3.12.	Customer report Z2	41

3.13.	Transaction log	42

4.	INI-file structure and settings	47

4.1.	Standard settings	47

4.2.	Advanced settings.	49

4.3.	Examples	51

5.	Directory structure	52

6.	Appendix A: Driver return codes	53

7.	Appendix B: ECR error codes	54

Introduction

SDRV is a command line tool for communicating with a new generation CHD ECRs (CHD3320, CHD5620 etc.). It provides a set of commands for ECR table (PLU, departments) and setting (date, time, option flags) modification, allows to reads various reports, set receipt graphical logo and much more. It also provides a fiscal printer mode commands. In short – if some action can be performed manually, it can also be performed using SDRV.

# Connecting ECR to PC

In order to communicate SDRV must be able to connect to ECR. Local ECR can be connected to a PC via a serial cable or via a standard network cable. Remote ECR should be connected to a network, which is accessible from the PC where SDRV runs.

## Serial cable connection

- Connect ECR to a PC using serial cable (see the picture below).

9-pin D-Sub Female

9-pin D-Sub Female

2

3

7

8

5

RX

TX

RTS

CTS

GND

RX

TX

RTS

CTS

GND

2

3

7

8

5

PC to ECR serial cable

- To test the connection use the following command:

SDRV.exe read 1 dept.csv /ComNumber=N

where N  is a PC serial port number used to connect to ECR.

## Ethernet connection

- Connect ECR to a PC or to a one of network devices (switch, hub, router, etc.).

- Configure ECR network settings (PRG-mode section 53).

- Generate ECR communication password (S-mode section 5).

- To test the connection use the following command:

SDRV.exe read 1 dept.csv  /IpAddress=192.168.1.102 /TcpPort=2000 /Password=12345

where /IpAddress, /TcpPort and /Password correspond to ECR configuration.

# Data preset commands

Driver usage:

SDRV.exe  command  data  filename  [parameters]

Command – action to perform on selected data.

| Command | Description |
| --- | --- |
| Read | Used to read ECR table records into an output file. |
| Send | Used to add/update ECR table with records from an input file. |
| Delete | Used to delete ECR table records. Record numbers are taken from an input file. |
| Clear | Used to delete clear ECR table (delete all records). |
| Init | Used to reinitialize ECR table (fill with default records) |
| ClearAndSend | Combines clear and send actions. Used to clear table and fill it with records from an input file. |

Data – ECR PGR-mode section number. Input / output file field sequence is the same as in corresponding PGR-mode section (see sub-sections below).

Filename – an input or an output file name (depending on the command).

Parameters – INI-file parameters in form /Name=Value. Used to override INI-file configuration.

## Departments

Data type: 1

Command line examples:

sdrv.exe send 1 dept.txt

sdrv.exe read 1 dept.txt

sdrv.exe delete 1 dept.txt

sdrv.exe clear 1

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … department count |
| 1 | Name | String | Max 20 symbols |
| 2 | Price | Decimal | 0.00 … 9999999.99 |
| 3 | Tax | Integer | 1 … number of taxes, 0 – non-taxable |
| 4 | HALO/LALO High / low price limits | Integer | 00 … 77 Digit 1 – number of high price limit digits, 0 – no limit Digit 2 – number of low price limit digits, 0 – no limit |
| 5 | Negative | Integer | 0 – disabled, 1 – enabled |
| 6 | Allow zero price | Integer | 0 – disabled, 1 – enabled |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 7 | Department Type | Integer | 1 – Normal department (default) 2 – Deposit (for countries which have deposits) 3 – Tare (for countries which have tare) |
| 8 | Single item | Integer | 0 – disabled (default), 1 – enabled |

File example:

15, "Dept 15", 4.88, 4, 0, 0, 0, 1

78, "Dept 78", 39.32, 3, 0, 0, 0, 1

88, "Dept 88", 36.26, 0, 0, 0, 0, 1

## PLUs

Data type: 3

Command line examples:

sdrv.exe send 3 plu.txt

sdrv.exe read 3 plu.txt

sdrv.exe delete 3 plu.txt

sdrv.exe clear 3

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | Max 13 digits |
| 1 | Name | String | Max 20 symbols |
| 2 | Price level 1 price | Decimal | 0.00 … 9999999.99 |
| 3 | Price level 2 price | Decimal | 0.00 … 9999999.99 |
| 4 | Price level 3 price | Decimal | 0.00 … 9999999.99 |
| 5 | Tax | Integer | 1 … number of taxes, 0 – non-taxable |
| 6 | Department | Integer | 0 … 99 |
| 7 | HALO/LALO High / low price limits | Integer | 00 … 77 Digit 1 – number of high price limit digits, 0 – no limit Digit 2 – number of low price limit digits, 0 – no limit |
| 8 | Negative item | Integer | 0 – disabled, 1 – enabled |
| 9 | Allow zero price | Integer | 0 – disabled, 1 – enabled |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 10 | PLU group | Integer | 1 … PLU group count, 0 – not used (default) |
| 11 | Allow price override | Integer | 0 – disabled, 1 – enabled |
| 12 | Quantity compulsory | Integer | 0 – disabled, 1 – enabled |

File example:

165, "PLU 165", 1.23, 4.56, 7.89, 1, 4, 0, 0, 0

2056, "PLU 2056", 1.23, 4.56, 7.89, 3, 65, 0, 0, 0

2567, "PLU 2567", 1.23, 4.56, 7.89, 4, 65, 0, 0, 0

## Linked PLUs

Data type: 4

Command line examples:

sdrv.exe send 4 linkedPlu.txt

sdrv.exe read 4 linkedPlu.txt

sdrv.exe delete 4 linkedPlu.txt

sdrv.exe clear 4

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Main PLU number | Integer | Max 13 digits |
| 1 | Linked PLU 1 number | Integer | Max 13 digits, 0 – not used |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 2 | Linked PLU 2 number | Integer | Max 13 digits, 0 – not used (default) |
| 3 | Linked PLU 3 number | Integer | Max 13 digits, 0 – not used (default) |
| 4 | Linked PLU 4 number | Integer | Max 13 digits, 0 – not used (default) |
| 5 | Linked PLU 5 number | Integer | Max 13 digits, 0 – not used (default) |

File example:

1, 22, 23

2, 73

## Tenders

Data type: 8

Command line examples:

sdrv.exe send 8 tender.txt

sdrv.exe read 8 tender.txt

sdrv.exe delete 8 tender.txt

sdrv.exe clear 8

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … tender count |
| 1 | Name | String | Max 20 symbols |
| 2 | Currency number | Integer | 0 … 99 |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 3 | Amount entry compulsory | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 4 | Over tender prohibited | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 5 | Under tender prohibited | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 6 | Tender type | Integer | 0 – payment with CASH, 1 – payment with CARD (by default 0) |
| 7 | Subtotal granularity | Decimal | 0.01 … 9999999.99 (by default 0.01) |
| 8 | Open cash drawer | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 9 | RA prohibited | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 10 | PO prohibited | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 11 | Disable negative amount in drawer | Integer | 0 – disabled, 1 – enabled (by default 0) |
| 12 | HALO/LALO | Integer | 00…XY, where X, Y - 0…7; X – HALO, Y - LALO |
| 13 | Cashback | Integer | 0 – disabled, 1 – enabled (by default 0) |

File example:

1, "CASH", 1

2, "CREDIT", 2

## Discounts / markups

Data type: 9

Command line examples:

sdrv.exe send 9 modifier.txt

sdrv.exe read 9 modifier.txt

sdrv.exe delete 9 modifier.txt

sdrv.exe clear 9

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … modifier count |
| 1 | Name | String | Max 20 symbols |
| 2 | Amount | Decimal | 0.00 … 9999999.99 |
| 3 | Percent rate | Decimal | 0.00 … 99.99 |
| 4 | Discount / markup | Integer | 0 – markup 1 – discount |
| 5 | Amount / percent rate | Integer | 0 – amount discount/markup 1 – percent discount/markup |
| 6 | Fixed / open | Integer | 0 – open (amount / rate must be entered) 1 – fixed (use preset amount / rate) |
| 7 | Modifier type | Integer | 1 – item. Applied to preceding item 2 – subtotal. Applied for current sale total (subtotal) 3 – item or subtotal, depending on the modifier’s position in a receipt. 4 – total modifier. Applied for a sale total. The total modifier can be registered anytime during a sale, it will be executed and printed at sale finalization (payment). |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 8 | PLU group | Integer | 1 – PLU group count, 0 – not used (default) |

File example:

1, "", 0.00, 0.00, 1, 1, 0, 1

2, "VIP discount", 0.00, 10.00, 1, 1, 1, 2

## Automatic promotions

Data type: 10

Command line examples:

sdrv.exe send 10 promotions.txt

sdrv.exe read 10 promotions.txt

sdrv.exe clear 10

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … automatic promotion count |
| 1 | Trigger condition | Integer | 1 – quantity 2 – amount |
| 2 | Modifier | Integer | 1 … modifier count The modifier should be a subtotal modifier. |
| 3 | Quantity | Decimal | 0.001 … 999999.999 0.000 – always apply modifier (default) |
| 4 | Amount | Decimal | 0.01 … 9999999.99 0.00 – always apply modifier (default) |
| 5 | PLU group | Integer | 1 … PLU group count, 0 – all items (default) |

File example:

1, 1, 7, 3, 0, 5

## ECR number

Data type: 12

Command line examples:

sdrv.exe send 12 EcrID.txt

sdrv.exe read 12 EcrID.txt

sdrv.exe clear 12

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | ECR number (ID) | String | Max 20 symbols |

## Option flags

Data type: 13

Command line examples:

sdrv.exe send 13 optionFlags.txt

sdrv.exe read 13 optionFlags.txt

sdrv.exe init 13

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Beep on key press | Integer | 0 – disabled, 1 – enabled |
| 2 | Join department and finacial reports | Integer | 0 – disabled, 1 – enabled |
| 3 | Print zeros in reports | Integer | 0 – disabled, 1 – enabled |
| 4 | Subtotal compulsory | Integer | 0 – disabled, 1 – enabled |
| 5 | Price level affects single item | Integer | 0 – disabled, 1 – enabled |
| 6 | Print header on journal for reports only | Integer | 0 – disabled, 1 – enabled |
| 7 | Receipt copy enabled | Integer | 0 – disabled, 1 – enabled |
| 8 | Reset receipt counter on Z1 report | Integer | 0 – disabled, 1 – enabled |
| 9 | Item modifiers affect PLU / department totals | Integer | 0 – disabled, 1 – enabled Currently this field is ignored! |
| 10 | Allow sale and refund in same transaction | Integer | 0 – disabled, 1 – enabled |
| 11 | Single line tender control transaactions | Integer | 0 – disabled, 1 – enabled |
| 12 | Print on journal in PRG mode | Integer | 0 – disabled, 1 – enabled |
| 13 | Receipt cut type | Integer | 0 – no cut 1 – full cut 2 – partial cut |
| 14 | Paper economic mode | Integer | 0 – disabled, 1 – enabled |
| 15 | Allow change in foreign currency | Integer | 0 – disabled, 1 – enabled |
| 16 | Clerk system | Integer | 0 – disabled, 1 – enabled |
| 17 | Print receipt at transaction end | Integer | 0 – disabled, 1 – enabled |
| 18 | Center text logo | Integer | 0 – disabled, 1 – enabled |
| 19 | Receipt printing in REG mode ON | Integer | 0 – disabled, 1 – enabled |
| 20 | Journal printing in X mode ON | Integer | 0 – disabled, 1 – enabled |
| 21 | REG-mode price entry input masking | Integer | 0 – disabled, 1 – enabled |
| 22 | Print Net grand total on Z1 report | Integer | 0 – disabled, 1 – enabled |
| 23 | Print rounded subtotal on receipt | Integer | 0 – disabled, 1 – enabled |
| 24 | Clerk auto log-out | Integer | 0 – disabled, 1 – enabled |
| 25 | Multiple single item receipts | Integer | 0 – disabled, 1 – enabled |
| 26 | Recalculate taxes on daily financial Z1 report | Integer | 0 – disabled, 1 – enabled |
| 27 | Compact tax print format | Integer | 0 – disabled, 1 – enabled |
| 28 | Print on receipt in PRG mode | Integer | 0 – disabled, 1 – enabled |
| 29 | Automatic Summer / Winter time change | Integer | 0 – disabled, 1 – enabled |
| 30 | Allow change in foreign currency in transaction payment currency only, e.g. if no payment in tender with specified currency, disable change in that currency | Integer | 0 – disabled, 1 – enabled |
| 31 | Defines price input behaviour | Integer | 0 – entered amount divided by 100 (12 will result in 0,12 added to transaction) 1 – entered amount multiplied by 100 (12 will result in 12,00 added to transaction) |
| 32 | Should cash drawer be opened on X reports | Integer | 0 – disabled, 1 – enabled |
| 33 | Shift key operation mode (All non-fiscal countries except CH) | Integer | 0 – Till next key press 1 – Till next shift key press 2 – Till transaction end |
| 34 | Hospitality | Integer | 0 – disabled, 1 – enabled |
| 35 | Should cash drawer be opened on Z reports | Integer | 0 – disabled, 1 – enabled |
| 36 | Show confirmation before customer EFT slip print. It is possible to choose to print EFT slip or to skip printing if confirmation is showed | Integer | 0 – disabled, 1 – enabled |
| 37 | Track number should be entered before starting transaction | Integer | 0 – disabled, 1 – enabled |
| 38 | Allow transaction browsing using list. If allowed (“1”) user can’t void lines partially, for example, if registered 1 PLU with discount, then it should be voided using list, not conventional registration | Integer | 0 – disabled, 1 – enabled |
| 39 | Normalize transaction on closing table and before printing receipt after tender | Integer | 0 – disabled, 1 – enabled |

## Date

Data type: 14

Command line examples:

sdrv.exe send 14 newDate.txt

sdrv.exe read 14 date.txt

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Current date | String | Format: YYYY.MM.DD |

File example:

"2005.12.01"

## Time

Data type: 15

Command line examples:

sdrv.exe send 15 newTime.txt

sdrv.exe read 15 time.txt

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Current time | String | Format: HH:MM:SS |

File example:

"12:05:04"

## Currencies

Data type: 16

Command line examples:

sdrv.exe send 16 currency.txt

sdrv.exe read 16 currency.txt

sdrv.exe clear 16

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … currency count |
| 1 | Name | String | Max 20 symbols |
| 2 | Exchange rate | Decimal | 0 … 99999.99999 |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 3 | Currency decimal count | Integer | 2 – default value |
| 4 | Exchange rate direction | Integer | 0 – foreign currency in local currency (default) 1 – local currency in foreign currency |

File example:

1, "LVL", 1.00000

2, "EUR", 0.70280

3, "USD", 0.48000

4, "LTL", 0.20400

5, "EEK", 0.04500

## Printing fonts and heights

Data type: 18

Command line examples:

sdrv.exe send 18 printOptions.txt

sdrv.exe read 18 printOptions.txt

sdrv.exe init 18

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Receipt height | Integer | 1 – half height 2 – normal height 3 – double height |
| 2 | Journal height | Integer | Same as 1 |
| 3 | Receipt font | Integer | 1 – text font 1 2 – text font 2 |
| 4 | Journal font | Integer | Same as 3 |
| 5 | Text logo height | Integer | Same as 1 |
| 6 | Footer height | Integer | Same as 1 |
| 7 | ECR serial number height | Integer | Same as 1 |
| 8 | Receipt number height | Integer | Same as 1 |
| 9 | Sale items height | Integer | Same as 1 |
| 10 | Sales total height | Integer | Same as 1 |
| 11 | Tendered amount height | Integer | Same as 1 |
| 12 | Change amount height | Integer | Same as 1 |
| 13 | Customer number height | Integer | Same as 1 |
| 14 | Customer name height | Integer | Same as 1 |

## Printing options

Data type: 19

Command line examples:

sdrv.exe send 19 printOptions.txt

sdrv.exe read 19 printOptions.txt

sdrv.exe init 19

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Print PLU code on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 2 | Print tax names and totals on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 3 | Print zero price on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 4 | Print subtotal amount on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 5 | Print no sale receipts | Integer | 0 – disabled, 1 – enabled |
| 6 | Print tax grand totals on financial report | Integer | 0 – disabled, 1 – enabled |
| 7 | Print graphical logo on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 8 | Print currency rate in reports | Integer | 0 – disabled, 1 – enabled |
| 9 | Print total in 2nd currency units on sales receips | Integer | 0 – disabled, 1 – enabled |
| 10 | Print in-store barcode quantity | Integer | 0 – disabled, 1 – enabled |
| 11 | Print ECR number | Integer | 0 – disabled, 1 – enabled |
| 12 | Print taxable amount on sales receips | Integer | 0 – disabled, 1 – enabled |
| 13 | Print taxable with tax amount on sales receips | Integer | 0 – disabled, 1 – enabled |
| 14 | Print tax amount on sales receips | Integer | 0 – disabled, 1 – enabled |
| 15 | Print foreign currency rate on sales receips | Integer | 0 – disabled, 1 – enabled |
| 16 | Print grand total on financial report | Integer | 0 – disabled, 1 – enabled |
| 17 | Print taxes on financial report | Integer | 0 – disabled, 1 – enabled |
| 18 | Print no-sale on financial report | Integer | 0 – disabled, 1 – enabled |
| 19 | Negative sales on reports | Integer | 0 – disabled, 1 – enabled |
| 20 | Print PC communication info on journal | Integer | 0 – disabled, 1 – enabled |
| 21 | Print graphical logo after paper cut | Integer | 0 – disabled, 1 – enabled |
| 22 | Print text logo after paper cut | Integer | 0 – disabled, 1 – enabled |
| 23 | Print error correct total on reports | Integer | 0 – disabled, 1 – enabled |
| 24 | Print negative transaction total on reports | Integer | 0 – disabled, 1 – enabled |
| 25 | Print return total on reports | Integer | 0 – disabled, 1 – enabled |
| 26 | Print modifier totals on reports | Integer | 0 – disabled, 1 – enabled |
| 27 | Print graphical logo on reports | Integer | 0 – disabled, 1 – enabled |
| 28 | Print total barcode on receipts | Integer | 0 – disabled, 1 – enabled |
| 29 | Print department turnover percent on reports | Integer | 0 – disabled, 1 – enabled |
| 30 | Print item count on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 31 | Print rounding on sales receipts | Integer | 0 – disabled, 1 – enabled |
| 32 | Print rounding difference amount on daily financial report | Integer | 0 – disabled, 1 – enabled |
| 33 | Print daily non-fiscal receipt count on daily financial report | Integer | 0 – disabled, 1 – enabled |
| 34 | Print receipt copy count on daily financial report | Integer | 0 – disabled, 1 – enabled |
| 35 | Print Daily X1 report count on daily financial report | Integer | 0 – disabled, 1 – enabled |
| 36 | Print short header (2 lines) on single item receipts | Integer | 0 – disabled, 1 – enabled |
| 37 | Print Plu barcode | Integer | 0 – disabled, 1 – enabled |

## COM port settings

Data type: 23

Command line examples:

sdrv.exe send 23 comPorts.txt

sdrv.exe read 23 comPorts.txt

sdrv.exe clear 23

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | COM port number | Integer | 1 … COM port count |
| 1 | Device type | Integer | 0 – none 1 – communications 2 – barcode scanner 3 – scale () 4 – MSR 5 – external printer 6 – fiscal memory 7 – scale (Pc100) 8 – scale (MK152TH21) 9 – scale (CasAp) 19 – modem (Insys336pro30) 20 – EFT terminal (Hypercom T4220) 21 – EFT terminal (Dancard I5100) |
| 2 | Baud rate | Integer | 1 – 2400 2 – 4800 3 – 9600 4 – 19200 5 – 38400 6 – 57600 7 – 115200 |
| 3 | Data bits | Integer | 1 – 8 bits 2 – 7 bits |
| 4 | Parity | Integer | 1 – none 2 – odd 3 – even |
| 5 | Stop bits | Integer | 1 – 1 stop bit 2 – 2 stop bits |
| 6 | Flow control | Integer | 1 – none 2 – hardware 3 – software |

File example:

1,1,5,1,2,1,1

2,2,3,1,1,1,1

## PLU presets

This table defines PLU number keyboard shortcuts (see ECR manual S-mode section 4 for keyboard programming details).

Data type: 24

Command line examples:

sdrv.exe send 24 presetPlu.txt

sdrv.exe read 24 presetPlu.txt

sdrv.exe clear 24

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Preset PLU number | Integer | 1 … preset PLU count |
| 1 | PLU number | Integer | Max 13 digits |

File example:

1, 1234567890123

2, 0000567890123

3, 1

## Clerks

Data type: 26

Command line examples:

sdrv.exe send 26 clerk.txt

sdrv.exe read 26 clerk.txt

sdrv.exe delete 26 clerk.txt

sdrv.exe clear 26

Mandatory fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … clerk count |
| 1 | Name | String | Max 20 symbols |
| 2 | Dallas key number | Binary | 12 hex digits, 0x00 – Dallas key is not used |
| 3 | Password | String | Max 20 symbols |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 4 | Clerk profile | Integer | 1 … clerk profile count, 0 – not used (default) |

File example:

1, “Clerk 01”, 0x00, “1q2W3e”

2, “Clerk 02”, 0x1FB35678C0D2, “”

## Clerk profiles

Data type: 27

Command line examples:

sdrv.exe send 27 clerkProfiles.txt

sdrv.exe read 27 clerkProfiles.txt

sdrv.exe clear 27

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … clerk profile count |
| 1 | Profile name | String | Max 20 symbols |
| 2 | S-mode access allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 3 | PRG-mode access allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 4 | REG-mode access allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 5 | X-mode access allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 6 | Z-mode access allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 7 | Refund allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 8 | Void allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 9 | Error correct allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 10 | Price shift allowed | Integer | 0 – not allowed, 1 – allowed (by default 0) |
| 11 | Training mode on | Integer | 0 – not allowed, 1 – allowed (by default 0) |

File example:

1, “Manager”, 1, 1, 1, 1, 1, 1, 1, 1, 1

2, “Operator”, 0, 0, 1, 0, 0, 0, 0, 1, 0

## Graphical logo

Data type: 29

Command line examples:

sdrv.exe send 29 logo.bmp

Logo sizes:

| ECR type | Width (fixed) | Max height |
| --- | --- | --- |
| CHD3320 | 288 | 288 |
| CHD5620 | 384 | 384 |

## Taxes

Data type: 30

Command line examples:

sdrv.exe send 30 tax.txt

sdrv.exe read 30 tax.txt

sdrv.exe clear 30

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … tax count |
| 1 | Percents rate | Decimal | 0 … 99.99 |
| 2 | Tax type | Integer | 0 – Non - taxable 1 – VAT (default) 2 – Add-on |
| 3 | Is active (PL only) | Integer | 0 – not active 1 – active (default for all countries except PL) |

File example:

1, 10.00, 1, 1

2, 5.00, 1, 1

3, 21.00, 1, 1

## Text header

Data type: 31

Command line examples:

sdrv.exe send 31 header.txt

sdrv.exe read 31 header.txt

sdrv.exe clear 31

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Header line | String | Max line length depends on a ECR printer width |

NOTE: Empty lines are not printed.

File example:

"<><><><><><><><><><><><><><><><><><>"

"            YOUR RECEIPT            "

"             THANK YOU              "

"<><><><><><><><><><><><><><><><><><>"

## Text footer

Data type: 32

Command line examples:

sdrv.exe send 32 footer.txt

sdrv.exe read 32 footer.txt

sdrv.exe clear 32

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Footer line | String | Max line length depends on a ECR printer width |

NOTE: Empty lines are not printed.

File example:

"             THANK YOU              "

## Customers

Data type: 34

Command line examples:

sdrv.exe send 34 customer.txt

sdrv.exe read 34 customer.txt

sdrv.exe delete 34 customer.txt

sdrv.exe clear 34

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | Max 16 digits |
| 1 | Info line 1 | String | Max 20 symbols |
| 2 | Info line 2 | String | Max 20 symbols |
| 3 | Info line 3 | String | Max 20 symbols |
| 4 | Info line 4 | String | Max 20 symbols |
| 5 | Modifier | Integer | 1 … modifier count |
| 6 | Percent rate | Decimal | 0.00 … 99.99 |

File example:

1, "John Smith", "Memorial Drive 17", "", "", 2, 10.00

## PLU barcode

Data type: 42

Command line examples:

sdrv.exe send 42 pluBarcode.txt

sdrv.exe read 42 pluBarcode.txt

sdrv.exe init 42

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Barcode type | Integer | 1 – FCCCCCvvvvvvL 2 – FCCCCCCvvvvvL 3 – FCCCCCCCvvvvL 4 – FFCCCCvvvvvvL 5 – FFCCCCCvvvvvL 6 – FFCCCCCCvvvvL, where F – fixed programmable digits (0...99), C – PLU cods v – changing part C – price checksum L – barcode checksum |
| 2 | Fixed programmable digits | Integer | 0 – 99 depending on barcode type |
| 3 | Decimal count of changing part | Integer | 0 – 6 depending on barcode type |

## Total barcode

Data type: 43

Command line examples:

sdrv.exe send 43 totalBarcode.txt

sdrv.exe read 43 totalBarcode.txt

sdrv.exe init 43

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Barcode type | Integer | 1 – AAAAAAAAvvvvL 2 – AAAAAAAvvvvvL 3 – AAAAAAvvvvvvL 4 - AAAAAAACvvvvL 5 – AAAAAACvvvvvL, where A – articul data (0...9), v – changing part C – price checksum L – barcode checksum |
| 2 | Articul | Integer | 0 – 99999999 depending on barcode type |
| 3 | Decimal count of changing part | Integer | 0 – 6 depending on barcode type |

## In store barcodes

This table defines special barcodes (barcode structure), which can be used by a shop to include an item price or weight into a barcode.

Data type: 44

Command line examples:

sdrv.exe send 44 barcodes.txt

sdrv.exe read 44 barcodes.txt

sdrv.exe clear 44

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 20 … 29 |
| 1 | Variable part type | Integer | 1 – PLU code 2 – Price 3 – Weight 4 – Price with check sum 5 – Weight with check sum |
| 2 | Variable part digit count | Integer | 4 … 5, 0 – no variable part |
| 3 | Variable part decimal count | Integer | 0 … 4 or 0 … 5, depending on field 1 |

Internal barcodes should be in the following format:  NNF…FV…VC

NN    – barcode number (2 digits)

F…F  – barcode fixed part digits

V…V – barcode variable part digits

C        – barcode check sum (1 digit)

Total barcode digit count should be 13.

If we use the below configuration barcode 207777771234X (X is a check digit) defines an item 207777770000X with an included price 12.34.

File example:

20, 2, 4, 2

21, 2, 4, 1

22, 2, 4, 0

23, 3, 4, 3

24, 3, 4, 2

25, 3, 4, 1

26, 1, 0, 0

27, 1, 0, 0

28, 2, 5, 2

29, 3, 5, 3

## Report arrangement

This table allows to define report lists to print.

Data type: 45

Command line examples:

sdrv.exe send 45 reportArrangements.txt

sdrv.exe read 45 reportArrangements.txt

sdrv.exe clear 45

Mandatory fields format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … report arrangement count |
| 1 | Report type 1 | Integer | 1 … 99 |
| 2 | Report type 2 | Integer | 1 … 99 |
| 3 | Report type 3 | Integer | 1 … 99 |
| 4 | Report type 4 | Integer | 1 … 99 |
| 5 | Report type 5 | Integer | 1 … 99 |

File example:

1, 3, 13, 0, 0, 0

2, 0, 0, 0, 0, 0

## Card ranges

This table assigns magnetic card ranges to card types (customer card, credit card etc.).

Data type: 47

Command line examples:

sdrv.exe send 47 cardRanges.txt

sdrv.exe read 47 cardRanges.txt

sdrv.exe clear 47

Mandatory fields format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … card range count |
| 1 | Range begin | Integer | 0 … 999999 |
| 2 | Range end | Integer | Range begin … 999999 |
| 3 | Card type | Integer | 1 – Customer 2 – Tender (credit card) |
| 4 | Tender number | Integer | 1 … tender count, 0 – no tender |

Optional fields:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 5 | Customer number | Integer | Max 16 digits, 0 – not used (default) |

File example:

1, 0001, 1000, 1, 0

2, 1001, 2000, 2, 2

3, 2001, 3000, 2, 3

## Paid out operation names

Data type: 48

Command line examples:

sdrv.exe send 48 PO.txt

sdrv.exe read 48 PO.txt

sdrv.exe clear 48

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … PO count |
| 1 | Operation name | String | Max 20 symbols |

File example:

1, "INVENTORY"

2, "LOTTERY"

## Receive on account operation names

Data type: 49

Command line examples:

sdrv.exe send 49 RA.txt

sdrv.exe read 49 RA.txt

sdrv.exe clear 49

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Number | Integer | 1 … RA count |
| 1 | Operation name | String | Max 20 symbols |

File example:

1, "RECEIVE ON ACCOUNT"

2, "LOTTERY"

## Macro key

Data type: 51

Command line examples:

sdrv.exe send 51 macro.txt

sdrv.exe read 51 macro.txt

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Macro ID | Integer | 1 … macro key count |
| 1…10 | Key value | Integer | See keyboard layout section field 1 |

File example:

1, 4001, 4002, 4003

2, 6049, 6048, 13001

## FTP settings

Data type: 52

Command line examples:

sdrv.exe send 52 Ftp.txt

sdrv.exe read 52 Ftp.txt

sdrv.exe clear 52

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Server IP address | String | XXX.XXX.XXX.XXX format string |
| 2 | User name | String | Max 20 symbols |
| 3 | Password | String | Max 20 symbols |
| 4 | FTP client mode | Integer | 0 – passive mode 1 – active mode Currently this field is ignored! Passive mode is used. |

Country specific fields (SG):

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 5 | Mall code | String | Max 20 symbols |
| 6 | Tenant code | String | Max 20 symbols |
| 7 | FTP report file type | Integer | 1 – type 1 2 – type 2 |

File example:

"192.168.001.055"

"john"

"12345"

0

## Ethernet settings

Data type: 53

Command line examples:

sdrv.exe send 53 Ethernet.txt

sdrv.exe read 53 Ethernet.txt

sdrv.exe clear 53

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | IP address | String | XXX.XXX.XXX.XXX format string |
| 2 | Subnet mask | String | Same as 1 |
| 3 | Gateway | String | Same as 1 |
| 4 | Port number | Integer | 1 … 65535 |

File example:

"192.168.001.001"

"255.255.255.000"

"192.168.001.254"

2000

## On-line protocol settings

Data type: 54

Command line examples:

sdrv.exe send 54 OnLine.txt

sdrv.exe read 54 OnLine.txt

sdrv.exe clear 54

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Server IP address | String | XXX.XXX.XXX.XXX format string |
| 2 | Server port number mask | Integer | 1 … 65535 |
| 3 | Data lookup order | Integer | 0 – local DB only (remote DB lookup is disabled) 1 – check local DB, if not found check remote DB 2 – check remote DB, if not found check local DB 3 – remote DB only |
| 4 | Send transaction log | Integer | 0 – do not send, 1 – send |

File example:

"255.255.255.000"

2222

1

0

## System settings

Data type: 82

Command line examples:

sdrv.exe read 82 System.txt

sdrv.exe send 82 System.txt

File record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Local currency decimal places | Integer | Read only |
| 2 | Transaction rounding | Integer | Not used. Always 0 |
| 3 | End rounding | Integer | Singapore version only |
| 4 | Date/time format | Integer | Not used. Always 0 |
| 5 | Language | Integer | 1 – 1st language 2 – 2nd language 3 – 3rd language |
| 6 | MAC address | Integer | Not used. Always 0 |
| 7 | Printing density | Integer | 1 – minimal density. 6 – maximal density. |
| 8 | S-mode password | String | Max 20 symbols |
| 9 | PRG-mode password | String | Max 20 symbols |
| 10 | Z-mode password | String | Max 20 symbols |
| 11 | ECR serial number | String | Max 16 symbols |
| 12 | Shop ID | String | Max 16 symbols |
| 13 | Electronic journal enabled | Integer | 0 – disabled, 1 – enabled |
| 14 | Electronic journal near full line count | Integer | 0 – electronic journal line count |
| 15 | Print receipt | Integer | 0 – disabled, 1 – enabled |
| 16 | Print journal | Integer | 0 – disabled, 1 – enabled |
| 17 | Transaction log storage | Integer | 0 – none 1 – memory 2 – file system (SD-card) 3 – both memory and file system |
| 18 | Electronic journal years to keep | Integer | 0 – not used |
| 19 | Reduced power mode time in secodns | Integer | 0 – not used |
| 20 | Minimum power mode time in secodns | Integer | 0 – not used |

File example:

2

1

0

0

2

0

1

""

""

""

""

""

0

500

1

1

0

0

600

300

## Keyboard layout

Data type: 84

Command line examples:

sdrv.exe send 84 keyboard.txt

sdrv.exe read 84 keyboard.txt

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Key ID | Integer | Format: LKKK L – One digit layout number (see table below) KKK – Three digit key number (0 – ECR key count) |
| 1 | Key value | Integer | Format: [F]FDDD [F]F – One or two digit function code (see table below) DDD – Three digit function data (see table below) |

Layout number:

| Value | Description |
| --- | --- |
| 1 | Main |
| 2 | Shift |
| 3 | Alpha |
| 4 | Alpha 2 |

Function code and data:

| Code | Description | Data |
| --- | --- | --- |
| 0 | None | 0 |
| 1 | Paper feed for joined receipt and journal printers | 0 |
| 2 | Receipt paper feed | 0 |
| 3 | Journal paper feed | 0 |
| 4 | Department | Department number |
| 5 | Macro | Macro number |
| 6 | Alpha-numeric character | Character |
| 7 | 2 zeros | 0 |
| 8 | 3 zeros | 0 |
| 9 | Price | 0 |
| 10 | Price level | 0 |
| 11 | Switch to "Alpha" layout | 0 |
| 12 | Error correct | 0 |
| 13 | Price modifier | Modifier number |
| 14 | Quantity | 0 |
| 15 | Received on account | RA number |
| 16 | Paid out | PO number |
| 17 | Switch to "Shift" layout | 0 |
| 18 | PLU | 0 |
| 19 | Preset PLU | Preset PLU number |
| 20 | Clerk | 0 |
| 21 | Return | 0 |
| 22 | Void | 0 |
| 23 | Clear | 0 |
| 24 | No sale | 0 |
| 25 | Subtotal | 0 |
| 26 | Tender | Tender number |
| 27 | Up arrow | 0 |
| 28 | Down arrow | 0 |
| 29 | Scale | 0 |
| 30 | Customer | 0 |
| 31 | Receipt copy | 0 |
| 32 | Keylock mode (CHD3030/CHD3050 ECRs) | 0 |
| 33 | Multi-character | Multi-character number |
| 34 | Double functionality key for RA and UP (arrow) | RA number |
| 35 | Double functionality key for PO and DOWN (arrow) | PO number |
| 36 | Currency | Currency number |
| 37 | Toggles receipt printing in REG mode | 0 |
| 38 | Switches to tax group (RCS) | Tax group number |
| 39 | Bottle item sale | 0 |
| 40 | Bottle item return | 0 |
| 41 | Invoice number entry | 0 |
| 42 | Enables customer discount logic for current transaction (RCS) | 0 |
| 43 | Open (start new or resume) if not started or close (suspend) started transaction | 0 |
| 44 | Display list (currently for REG mode transaction) UNUSED currently | 0 |
| 45 | Start print open track report | 0 |
| 46 | Print bill for specified table | 0 |
| 47 | Changes taxes for items in transaction | 0 |
| 48 | Starts internal barcode scanner | 0 |
| 49 | Starts track (table) transfer sequence | 0 |

File example:

1000,2000

1001,3000

1002,11000

1003,4005

1004,4010

## Service reminder

Data type: 95

Command line examples:

sdrv.exe send 95 serviceReminder.txt

sdrv.exe read 95 serviceReminder.txt

sdrv.exe init 95

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Display message text | String | Max 20 symbols |
| 2 | Display start date | String | Format: YYYY.MM.DD |
| 3 | Printer message text | String | Max 20 symbols |
| 4 | Print start date | String | Format: YYYY.MM.DD |

File example:

"CALL SERVICE",

“2010.02.01”,

“CALL SERVICE”,

“2010.03.01”

# Report commands

Driver usage:

SDRV.exe  command  report  filename  [parameters]

Command – action to perform on selected report.

| Command | Description |
| --- | --- |
| Read | Used to read ECR report into an output file. |
| Clear | Used to clear ECR report data. |
| ReadAndClear | Combines read and clear actions. Used to read and clear ECR report data (similar to Z-report). |

NOTE: Clear command is allowed only if corresponding data has been read and where were no transactions in between Read and Clear. Otherwise driver will return “command error” return code (see Appendix A).

Report – a report number. Driver report numbers agree with ECR report numbers plus 100. For example, if ECR daily financial report number is 1 then the driver’s daily financial report number will be 101.

Filename – an output file name.

Parameters – INI-file parameters in form /Name=Value. Used to override INI-file configuration.

## Daily financial report Z1

Report number: 101

Command line examples:

sdrv.exe read 101 sales_Z1.txt

sdrv.exe clear 101

NOTE: Clear command for daily financial Z1 report is disabled at all for most of countries due to fiscal regulations.

Daily financial report consists of multiple record groups. First record field specifies the group; other record fields contain record data.

Record groups:

| Value | Description |
| --- | --- |
| 1 | Tax counters |
| 2 | Sales counters |
| 3 | Tender counters |
| 4 | Receive on account counters |
| 5 | Paid out counters |
| 6 | Discount/mark-up counters |
| 7 | Sales grand total counters |
| 8 | Tax grand total counters |
| 9 | Correction counters |
| 10 | No sale counters |

Tax counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 1 |
| 1 | Tax number | Integer | 1 … tax count, 0 – non-taxable |
| 2 | Tax | Decimal | –999999999999.99 … 999999999999.99 |
| 3 | Taxable | Decimal | –999999999999.99 … 999999999999.99 |
| 4 | Taxable with tax | Decimal | –999999999999.99 … 999999999999.99 |

Sales counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 2 |
| 1 | Counter number | Integer | 1 … counter count (see table below) |
| 2 | Operation count | Decimal | 0 … 99999999 |
| 3 | Amount | Decimal | –999999999999.99 … 999999999999.99 |

Counter number:

| Value | Description |
| --- | --- |
| 1 | Gross sales |
| 2 | Net sales |
| 3 | Negative items (departments/PLU) |
| 4 | Negative transactions |

Tender counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 3 |
| 1 | Tender number | Integer | 1 … tender count |
| 2 | Operation count | Integer | 0 … 99999999 |
| 3 | Sales amount in tender currency | Decimal | –999999999999.99 … 999999999999.99 |
| 4 | Sales amount in local currency | Decimal | –999999999999.99 … 999999999999.99 |
| 5 | In drawer amount in tender currency | Decimal | –999999999999.99 … 999999999999.99 |
| 6 | In drawer amount in local currency | Decimal | –999999999999.99 … 999999999999.99 |

Receive on account counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 4 |
| 1 | Received on account number | Integer | 1 … received on account count |
| 2 | Tender number | Integer | 1 … tender count |
| 3 | Operation count | Integer | 0 - 99999999 |
| 4 | Amount in tender currency | Decimal | –999999999999.99 … 999999999999.99 |
| 5 | Amount in local currency | Decimal | –999999999999.99 … 999999999999.99 |

Paid out counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 5 |
| 1 | Paid out number | Integer | 1 … paid out count |
| 2 | Tender number | Integer | 1 … tender count |
| 3 | Operation count | Integer | 0 … 99999999 |
| 4 | Amount in tender currency | Decimal | –999999999999.99 … 999999999999.99 |
| 5 | Amount in local currency | Decimal | –999999999999.99 … 999999999999.99 |

Modifier (discount/markup) counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 6 |
| 1 | Modifier number | Integer | 1 … modifier count |
| 2 | Operation count | Integer | 0 … 99999999 |
| 3 | Amount | Decimal | –999999999999.99 … 999999999999.99 |

Sales grand total counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 7 |
| 1 | Counter number | Integer | Always 1 |
| 2 | Gross sales amount | Decimal | –999999999999.99 … 999999999999.99 |
| 3 | Net sales amount | Decimal | –999999999999.99 … 999999999999.99 |

Tax grand total counter record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 8 |
| 1 | Tax number | Integer | 1 … tax count, 0 – non-taxable |
| 2 | Tax amount | Decimal | –999999999999.99 … 999999999999.99 |

Correction record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 9 |
| 1 | Counter number | Integer | 1 … counter count (see table below) |
| 2 | Operation count | Integer | 0 … 99999999 |
| 3 | Amount | Decimal | –999999999999.99 … 999999999999.99 |

Counter number:

| Value | Description |
| --- | --- |
| 1 | Item void |
| 2 | Modifier void |
| 3 | Error correction |
| 4 | Refunds |
| 5 | Cancelled transactions |

Misc. counters record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Record type | Integer | Always 10 |
| 1 | Counter number | Integer | 1 … counter count (see table below) |
| 2 | Operation count | Integer | 0 … 99999999 |

Counter number:

| Value | Description |
| --- | --- |
| 1 | No sale |
| 2 | Receipt copy count |
| 3 | Daily financial X reports |

File example:

Tax records

1, 1, 0.92, 5.08, 6.00

1, 2, 0.00, 0.00, 0.00

1, 3, 0.00, 0.00, 0.00

1, 4, 0.00, 0.00, 0.00

1, 0, 0.00, 0.00, 1.23

Sales records

2, 1, 1, 7.23

2, 2, 1, 7.23

2, 3, 0, 0.00

Tender records

2, 4, 0, 0.00

3, 1, 1, 2.00, 2.00, 0.20, 0.20

3, 2, 1, 7.44, 5.23, 10.00, 7.03

3, 3, 0, 0.00, 0.00, 0.00, 0.00

3, 4, 0, 0.00, 0.00, 0.00, 0.00

Received on account records

4, 1, 1, 1, 2.00, 2.00

4, 1, 2, 1, 7.44, 5.23

4, 1, 3, 0, 0.00, 0.00

4, 1, 4, 0, 0.00, 0.00

4, 2, 1, 0, 0.00, 0.00

4, 2, 2, 0, 0.00, 0.00

4, 2, 3, 0, 0.00, 0.00

4, 2, 4, 0, 0.00, 0.00

4, 3, 1, 0, 0.00, 0.00

4, 3, 2, 0, 0.00, 0.00

4, 3, 3, 0, 0.00, 0.00

4, 3, 4, 0, 0.00, 0.00

Paid out records

5, 1, 1, 1, 2.00, 2.00

5, 1, 2, 1, 7.44, 5.23

5, 1, 3, 0, 0.00, 0.00

5, 1, 4, 0, 0.00, 0.00

5, 2, 1, 0, 0.00, 0.00

Paid out records

5, 2, 2, 0, 0.00, 0.00

5, 2, 3, 0, 0.00, 0.00

5, 2, 4, 0, 0.00, 0.00

5, 3, 1, 0, 0.00, 0.00

5, 3, 2, 0, 0.00, 0.00

5, 3, 3, 0, 0.00, 0.00

5, 3, 4, 0, 0.00, 0.00

Modifier records

6, 1, 0, 0.00

6, 2, 0, 0.00

6, 3, 0, 0.00

6, 4, 0, 0.00

6, 5, 0, 0.00

6, 6, 0, 0.00

6, 7, 0, 0.00

6, 8, 0, 0.00

6, 9, 0, 0.00

Sales grand total record

6, 10, 0, 0.00

7, 1, 7.23, 7.23

Tax grand total records

8, 1, 0.92

8, 2, 0.00

8, 3, 0.00

8, 4, 0.00

8, 0, 0.00

Correction records

9, 1, 0, 0.00

9, 2, 0, 0.00

9, 3, 0, 0.00

9, 4, 0, 0.00

9, 5, 0, 0.00

Misc counters

10, 1, 0

10, 2, 0

10, 3, 0

## Daily financial report Z2

Report number: 102

Command line examples:

sdrv.exe read 102 sales_Z2.txt

sdrv.exe clear 102

Data format: the same as in daily financial Z1 report.

## Department report Z1

Report number: 103

Command line examples:

sdrv.exe read 103 dept_Z1.txt

sdrv.exe clear 103

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Department number | Integer | 1 … department count |
| 1 | Department name | String | Max 20 symbols |
| 2 | Sales quantity | Decimal | 0 … 999999.999 |
| 3 | Sales amount | Decimal | –999999999999.99 … 999999999999.99 |

File example:

1, "DEPT 01", 3.245, 3.12

2, "DEPT 02", 4.000, 2.00

3, "DEPT 03", 3.000, 0.75

## Department report Z2

Report number: 104

Command line examples:

sdrv.exe read 104 dept_Z2.txt

sdrv.exe clear 104

Data format: the same as in department Z1 report.

## PLU report Z1

Report number: 105

Command line examples:

sdrv.exe read 105 plu_Z1.txt

sdrv.exe clear 105

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | PLU number | Integer | 1 … 1014 – 1 |
| 1 | Name | String | Max 20 symbols |
| 2 | Sales quantity | Decimal | 0 … 999999.999 |
| 3 | Sales amount | Decimal | –999999999999.99 … 999999999999.99 |

File example:

1, "PLU 1", 2.000, 2.46

2, "PLU 2", 1.000, 1.25

## PLU report Z2

Report number: 106

Command line examples:

sdrv.exe read 106 plu_Z2.txt

sdrv.exe clear 106

Data format: the same as in PLU Z1 report.

## Clerk report Z1

Report number: 108

Command line examples:

sdrv.exe read 108 clerk_Z1.txt

sdrv.exe clear 108

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Clerk number | Integer | 1 … clerk count |
| 1 | Counter number | Integer | 1 … counter count (see table below) |
| 2 | Operation count | Decimal | 0 … 99999999 |
| 3 | Amount | Decimal | –999999999999.99 … 999999999999.99 |

Counter number:

| Value | Description |
| --- | --- |
| 1 | Net sales |
| 2 | Received on account total |
| 3 | Paid out total |
| 4 | Discount total |
| 5 | Add-on total |
| 6 | Item void |
| 7 | Not used! |
| 8 | Error correction |
| 9 | Refunds |
| 10 | Cancelled transactions |
| 11 | Negative items (departments/PLU) |

File example:

1, 1, 28, 152.461

1, 2, 1, 500.00

## Clerk report Z2

Report number: 109

Command line examples:

sdrv.exe read 109 clerk_Z2.txt

sdrv.exe clear 109

Data format: the same as in clerk Z1 report.

## Hourly report

Report number: 112

Command line examples:

sdrv.exe read 112 hourly.txt

sdrv.exe clear 112

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Hour | Integer | 1 … 24 (1 means 00:00 – 00:59) |
| 1 | Operation count | Integer | 0 … 99999999 |
| 2 | Sales amount | Decimal | –999999999999.99 … 999999999999.99 |

File example:

11, 28, 152.46

12, 42, 163.25

13, 36, 231.25

## In drawer report

Report number: 113

Command line examples:

sdrv.exe read 113 in_drawer.txt

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Tender number | Integer | 1 … tender count |
| 1 | Tender name | String | Max 20 symbols |
| 2 | Amount in tender currency | Decimal | –999999999999.99 … 999999999999.99 |
| 3 | Amount in local currency | Decimal | –999999999999.99 … 999999999999.99 |

File example:

1, "CASH LVL", 3.00, 3.00

2, "CASH EUR", 5.25, 3.69

## Customer report Z1

Report number: 136

Command line examples:

sdrv.exe read 136 customer_Z1.txt

sdrv.exe clear 136

Data format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 0 | Customer number | Integer | 1 … 1015 – 1 |
| 1 | Operation count | Decimal | 0 … 99999999 |
| 2 | Sales amount | Decimal | –999999999999.99 … 999999999999.99 |

File example:

1000123, 6, 152.461

1000124, 2, 50.00

## Customer report Z2

Report number: 137

Command line examples:

sdrv.exe read 137 customer_Z2.txt

sdrv.exe clear 137

Data format: the same as in customer Z1 report.

## Transaction log

Command number: 200

Command line examples:

sdrv.exe read 200 trn_log.txt

sdrv.exe clear 200

Transaction log consists of multiple records. First record field specifies the record type; other fields contain record data.

Transaction log header record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 0 |
| 2 | Transaction log version | Number | Currently 2 or 3 |
| 3 | ECR serial number | String |  |
| 4 | Shop Id | String |  |
| 5 | ECR number | String |  |
| 6 | Transaction log report number | Number | Currently not used (always 0). |

Example:

0, 3, "12345678","Test shop","11122",0

Transaction header record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 1 |
| 2 | Transaction type | Number | 1 – retail 2 – tender control 3 – no-sale |
| 3 | Receipt number | Number |  |
| 4 | Cancelled flag | Number | 1 – cancelled, 0 – not cancelled |
| 5 | Transaction start time | String | Format: YYYY.MM.DD HH:MM:SS |
| 6 | Transaction end time | String | Format: YYYY.MM.DD HH:MM:SS |
| 7 | Clerk number | Number | 0 means no clerk |

Example:

1,1,2,0,"04.05.2008 12:31”,"04.05.2007 12:32"

Department record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 2 |
| 2 | Department number | Number |  |
| 3 | Department name | String | Starting from version 3 |
| 4 | Tax | Number |  |
| 5 | Quantity | Number | Signed quantity (affected by void and return flags) |
| 6 | Unit price | Number | Signed unit price (affected by negative flag) |
| 7 | Printed amount | Number | Amount printed on the receipt |
| 8 | Actual amount | Number | Amount with price modifiers included (subtotal or/and item dicounts etc.) |
| 9 | Return flag | Number | 1 – return, 0 – sale |
| 10 | Void flag | Number | 1 – void, 0 – non-void |
| 11 | Negative flag | Number | 1 – negative item, 0 – normal item |
| 12 | Price source | Number | 0 – manual price entry, 1 – department price |

Example:

2,1,1,2.000,1.23,2.46,2.46,0,0,0

Department with item name record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 15 |
| 2 | Department number | Number |  |
| 3 | Item name | String |  |
| 4 | Tax | Number |  |
| 5 | Quantity | Number | Signed quantity (affected by void and return flags) |
| 6 | Unit price | Number | Signed unit price (affected by negative flag) |
| 7 | Printed amount | Number | Amount printed on the receipt |
| 8 | Actual amount | Number | Amount with price modifiers included (subtotal or/and item dicounts etc.) |
| 9 | Return flag | Number | 1 – return, 0 – sale |
| 10 | Void flag | Number | 1 – void, 0 – non-void |
| 11 | Negative flag | Number | 1 – negative item, 0 – normal item |
| 12 | Price source | Number | 0 – manual price entry, 1 – department price |

Example:

2,1,”Item #123”,1,2.000,1.23,2.46,2.46,0,0,0

PLU record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 3 |
| 2 | PLU number | Number |  |
| 3 | PLU name | String | Starting from version 3 |
| 4 | Tax | Number |  |
| 5 | Quantity | Number | Signed quantity (affected by void and return flags) |
| 6 | Unit price | Number | Signed unit price (affected by negative flag) |
| 7 | Printed amount | Number | Amount printed on the receipt |
| 8 | Actual amount | Number | Amount with price modifiers included (subtotal or/and item dicounts etc.) |
| 9 | Return flag | Number | 1 – return, 0 – sale |
| 10 | Void flag | Number | 1 – void, 0 – non-void |
| 11 | Negative flag | Number | 1 – negative item, 0 – normal item |
| 12 | Price source | Number | 0 – manual price entry, 1,2, … – PLU price level |

Example:

3,1,5,2.000,1.23,2.46,2.46,0,0,0

Item modifier record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 4 |
| 2 | Modifier number | Number |  |
| 3 | Discount flag | Number | 1 – discount, 0 – add-on |
| 4 | Percent rate flag | Number | 1 – percent modifier, 0 – amount modifier |
| 5 | Amount | Number |  |
| 6 | Percent rate | Number |  |

Example:

4,2,1,1,-1.23,10.00

Subtotal modifier record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 5 |
| 2 | Modifier number | Number |  |
| 3 | Discount flag | Number | 1 – discount, 0 – add-on |
| 4 | Percent rate flag | Number | 1 – percent modifier, 0 – amount modifier |
| 5 | Amount | Number |  |
| 6 | Percent rate | Number |  |

Example:

5,3,1,1,-5.53,10.00

Customer modifier record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 6 |
| 2 | Customer number | Number |  |
| 3 | Modifier number | Number |  |
| 4 | Discount flag | Number | 1 – discount, 0 – add-on |
| 5 | Percent rate flag | Number | 1 – percent modifier, 0 – amount modifier |
| 6 | Amount | Number |  |
| 7 | Percent rate | Number |  |

Example:

6,113681159209,4,1,1,-1.23,10.00

Subtotal record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 7 |
| 2 | Tender number | Number |  |
| 3 | Currency number | Number |  |
| 4 | Displayed amount | Number |  |
| 5 | Total Flag | Number |  |

Example:

7,1,1,15.27,0

Tender record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 8 |
| 2 | Tender number | Number |  |
| 3 | Currency number | Number |  |
| 4 | Tender amount in local currency | Number |  |
| 5 | Tender amount | Number |  |

Example:

8,1,1,15.27,15.27

Comment record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 10 |
| 2 | Comment text | String |  |

Example:

10,”Some comment”

Receive on account record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 11 |
| 2 | Tender number | Number |  |
| 3 | Count | Number |  |
| 4 | Amount | Number |  |

Example:

11,1,5,10.00

Paid out record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 12 |
| 2 | Tender number | Number |  |
| 3 | Count | Number |  |
| 4 | Amount | Number |  |

Example:

12,2,1,12.25

Customer record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 13 |
| 2 | Customer number | Number |  |

Example:

13,113681159209

Tax record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 14 |
| 2 | Tax number | Number |  |
| 3 | Taxable with tax | Number |  |
| 4 | Taxable amount | Number |  |
| 5 | Tax amount | Number |  |

Example:

14,1,13.68,11.59,2.09

Total modifier registration record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 16 |
| 2 | Modifier number | Number |  |
| 3 | Discount flag | Number | 1 – discount, 0 – add-on |
| 4 | Percent rate flag | Number | 1 – percent modifier, 0 – amount modifier |
| 5 | Amount | Number | Amount or 0 if modifier is a percent modifier |
| 6 | Percent rate | Number | Rate or 0 if modifier is an amount modifier |

Example:

15,4,1,1,0,10.00

Total modifier record format:

| № | Description | Type | Value |
| --- | --- | --- | --- |
| 1 | Record type | Number | Always 17 |
| 2 | Modifier number | Number |  |
| 3 | Discount flag | Number | 1 – discount, 0 – add-on |
| 4 | Percent rate flag | Number | 1 – percent modifier, 0 – amount modifier |
| 5 | Amount | Number |  |
| 6 | Percent rate | Number |  |

Example:

16,4,1,1,11.30,10.00

# INI-file structure and settings

SDRV.ini file contains driver configuration parameters. It consists of a single [Common] section with common parameters and multiple [ECR_n] sections with ECR specific parameters. Parameters from [ECR_n] section override ones from [Common] section.

In case of single [ECR_n] section it is used by default. In case of multiple [ECR_n] sections a section to use must be specified. This can be done by /ECR=n command line parameter or by “DefaultECR” [Common] section parameter.

NOTE: Configuration parameters can also be specified via command line (see driver usage) and if so will override INI-file settings.

## Standard settings

DefaultECR

[ECR_n] section number to use.

DbgErrorLog = 0…3

0 = do not write debug info (default)

1 = write debug info only in case of unrecoverable error

2 = write debug info in case of any error

3 = always write debug messages

Delimiter = any character

CSV-file field delimiter. By default is ‘,’.

IgnoreDuplicates = 0…1

Ignore input file records with duplicating numbers, i.e. do not interpret them as a syntax error.

By default is 0.

[ECR_n] section

ComNumber

(WINDOWS) PC serial port number. Used when ECR is connected via a serial port.

TTY

(LINUX) PC serial port, tty name. Used when ECR is connected via a serial port.

IpAddress

ECR IP address. Used then ECR is connected via a network.

TcpPort

ECR TCP port. Used then ECR is connected via a network.

Password

Password used to authorize a communication session. Password is mandatory if ECR is connected via a network.

## Advanced settings.

All settings below are optional. By default they are set to optimal values. But in certain situations advanced users (for example service personnel) may want to change these settings.

StandardLog = 0…1

Use or do not use the standard log file. Standard log file contain started, finished, statistics and also error messages. By default is 1.

EcrCommMaxTime

Maximal byte transfer time between ECR and PC in milliseconds. By default 100ms for local ECR and 1000ms for network ECR.

EcrOperationMaxTime

Maximal ECR operation (command processing) time in milliseconds. By default 2000ms.

EcrRecoveryTime

Sleep time in milliseconds after a communication error. By default 2000ms.

EcrDisconnectTime

Sleep time in milliseconds after disconnecting from ECR. By default 0ms.

RetryCount

Retry count on communication errors. By default 1.

ComSettings

Additional serial port configuration in windows “mode” utility format (for details see Windows help). By default "baud=38400 parity=o data=8 stop=1 rts=hs dtr=hs octs=on".

SerialProtocolRetryCount

Retry count on communication errors (serial port low-level protocol). By default 5.

SerialProtocolResponseMaxTime

Maximal response time in milliseconds (serial port low-level protocol). By default 1000ms.

RecordsInPacket

When sending data specifies number of records to be put into a single packet. By default 1.

In case of slow Ethernet connection this can significantly decrease data send time.

FileEncoding

Specifies CSV file encoding which will be converted to EcrEncoding when file will be send to ECR. If data is read from ECR, parameter specifies in which encoding CSV file with received data must be saved. Can be used only together with EcrEncoding

EcrEncoding

Specifies encoding used by ECR. When data is send to ECR, parameter specifies to which encoding data must be converted before sending it to ECR. When data is read from ECR, parameter specifies from which encoding data must be converted.

## Examples

Simple INI –file for ECR connected via serial port:

[ECR_1]

ComNumber = 10

Simple INI –file for ECR connected via Ethernet:

[ECR_1]

IpAddress = 159.148.235.12

TcpPort = 2000

Password = 1234023406

# Directory structure

Location of configuration and log files depends on the operating system used.

By default:

- Windows version of the driver reads configuration INI-file from the directory, where the driver is located. The same directory is used to write standard and debug log files.

- Linux version of the driver reads configuration INI-file from directory /etc/sdrv. Standard and debug log files are written to /var/log/sdrv (driver has to be granted write access to these files).

It is also possible to change log file location by settings LogFilePath property.

LogFilePath = ”C:\Log Files\SDRV\log_1.txt”

# Appendix A: Driver return codes

| Error code | Description |
| --- | --- |
| 0 | OK |
| 1 | Command line error |
| 2 | Input file contains badly formated, duplicate or empty records |
| 3 | ECR is busy |
| 4 | Command can not be executed in current context |
| 5 | Communication error |
| 10 | Configuration error (bad or missing parameter) |
| 11 | Authorization failed (bad password) |
| 12 | Protocol is not activated |
| 13 | File open/read/write error |
| 14 | Some input file records were rejected by ECR |
| 15 | Unknown command |
| 50 | System error |
| 99 | Internal driver error |

# Appendix B: ECR error codes

Business logic errors

| Error code | Description |
| --- | --- |
| 1 | No clerk |
| 2 | Clerk key error |
| 3 | Invalid password |
| 4 | Z report not issued |
| 11 | Bad input sequence |
| 12 | Operation prohibited |
| 13 | Key-lock error |
| 14 | Department not found |
| 15 | PLU not found |
| 16 | PLU preset not found |
| 17 | Modifier not found |
| 18 | Customer not found |
| 19 | Tax not found |
| 31 | Subtotal required |
| 32 | Tender not found |
| 33 | Currency not found |
| 34 | Over tender |
| 35 | Under tender |
| 36 | Amount compulsory |
| 37 | RA prohibited |
| 38 | PO prohibited |
| 39 | Not enough money in drawer |
| 55 | High amount |
| 56 | Low amount |
| 57 | Amount too big |
| 58 | Negative balance |
| 59 | Clerk system is OFF |
| 72 | Transaction has too many lines |
| 74 | Transaction log is full |
| 76 | PC comm. report clear in progress |
| 77 | Data locked |
| 80 | Operation failed |
| 81 | Bad card |
| 82 | Card expired |
| 99 | Internal ECR error |

Hardware errors

| Error code | Description |
| --- | --- |
| 20 | Generic printer error |
| 21 | Receipt paper end |
| 22 | Journal paper end |
| 23 | Receipt printer head up |
| 24 | Journal printer head up |
| 25 | Receipt printer overheat |
| 26 | Journal printer overheat |
| 27 | Receipt printer is disconnected |
| 28 | Journal printer is disconnected |
| 20 | Printer voltage error |
| 29 | Printer cutter error |
| 40 | Fiscal memory read error |
| 41 | Fiscal memory not connected |
| 42 | Fiscal memory write error |
| 43 | Fiscal memory is full |
| 50 | Electronic journal write error |
| 51 | Electronic journal near full |
| 52 | Electronic journal full |
| 60 | Generic display error |